The Western Digital Corporation's

Field Applications Engineering

Document for WDTYPES.EXE Utility

(C) 1993 Western Digital Corp.

This document is intended to accompany the WDTYPES.EXE utility

placed on this BBS for your convenience. First our disclaimer:

Western Digital assumes no liability for the fitness, operation

or results of this program. It was created solely for systems

integrators' use in configuring an AT compatible computer.

Now that that's out of the way, we shall go on ...

This utility is intended to aid beginners or those stuck with

undocumented equipment in the installation of Western Digital's

hard disk controllers and / or drives in most AT compatible

computers.

When loaded, the utility will scan the entire 64K page of BIOS

memory for a configuration that matches the original AT's

drive type #1. This drive was defined as follows :

306 cylinders

4   heads

17  sectors per track

This specific signature is searched throughout the BIOS area.

If found, the utility calls this a drive type 1, and proceeds

to display all other valid drive type tables. Many BIOS

companies have nonsensical ones included in a string of valid

configurations. The program is trained to ignore those types

that have zero heads or cylinders, and to stop the search when

a larger than 2047 cylinder or 16 head type is found.

In the event a match for the 10 Megabyte drive test fails, the

program will search for a 20 Megabyte drive of the following

configuration :

615 cylinders

4   heads

17  sectors per track

Several versions of BIOS were distributed with this configuration

as the first drive type.

IN CASE OF DIFFICULTY ...

To verify the findings of this utility, run your system's

SETUP utility and define a hard drive type 1. Your hard drive

can then be installed ( ... if it has at least 4 heads and 306

cylinders ...) and it should function as a 10 Megabyte hard

drive. The DOS utility FDISK should report 305 ( or 304 )

cylinders.

REMEMBER ...

You can only use a drive type that meets the following :

1) LESS than or EQUAL TO the actual number of

CYLINDERS on the drive you are trying to

install.

2) EQUAL TO the actual number of HEADS on the

drive you are trying to install.

3) EQUAL to the SECTORS PER TRACK of BOTH THE

DRIVE AND CONTROLLER !!

ST-506 MFM DRIVES & CONTROLLERS HAVE 17 SECTORS

ST-506 RLL DRIVES & CONTROLLERS HAVE 26-31 SECTORS *

ESDI DRIVES AND CONTROLLERS HAVE 17-54 SECTORS  *

* Western Digital manufactures controllers that have ON-BOARD

BIOS ROMS to accept other than motherboard BIOS drive

configurations. Contact your dealer.

Below is an example of the program in operation:

Scanning system BIOS for an AT drive type 1 ... PLEASE STAND BY ...

TYPE    CYLINDERS    HEADS     PRECOMP     LAND ZONE    SECTORS      MEG

1         306         4         128          305         17          10

2         615         4         300          615         17          20

3         615         6         300          615         17          30

4         940         8         512          940         17          62

5         940         6         512          940         17          46

6         615         4         65535        615         17          20

7         462         8         256          511         17          30

8         733         5         65535        733         17          30

9         900         15        65535        901         17          112

10        820         3         65535        820         17          20

11        855         5         65535        855         17          35

12        855         7         65535        855         17          49

13        306         8         128          319         17          20

14        733         7         65535        733         17          42

Press space bar to see other drive types ...

TYPE    CYLINDERS    HEADS     PRECOMP     LAND ZONE    SECTORS      MEG

15        0           0          0            0           0          0

16        612         4          0            663         17         20

17        977         5          300          977         17         40

18        977         7          65535        977         17         56

19        1024        7          512          1023        17         59

20        733         5          300          732         17         30

21        733         7          300          732         17         42

22        733         5          300          733         17         30

23        306         4          0            336         17         10

24        0           0          0            0           0          0

25        615         4          0            615         17         20

26        1024        4          65535        1023        17         34

27        1024        5          65535        1023        17         42

28        1024        8          65535        1023        17         68

29        512         8          256          512         17         34

30        615         2          615          615         17         10

Press space bar to see other drive types ...

TYPE    CYLINDERS    HEADS     PRECOMP     LAND ZONE    SECTORS      MEG

31        989         5          0           989          17         41

32        1020        15         65535       1024         17         127

33        0           0          0           0            0          0

34        0           0          0           0            0          0

35        1024        9          1024        1024         17         76

36        1024        5          512         1024         17         42

37        830         10         1792        830          17         68

38        820         10         65535       821          17         68

39        615         4          128         664          17         20

40        615         8          128         664          17         40

41        917         15         65535       918          17         114

42        1023        15         65535       1024         17         127

43        823         10         512         823          17         68

44        820         6          65535       820          17         40

45        1024        8          65535       1024         17         68

46        925         9          65535       925          17         69

Press space bar to see other drive types ...

TYPE    CYLINDERS    HEADS     PRECOMP     LAND ZONE    SECTORS      MEG

47        699         7          256         700          17         40

End of drive tables ... press space bar to return to system ...

You merely select the type number that best matches your

hard drive, and run your manufacturer's SETUP program.

The next step is to run a LOW-LEVEL FORMAT utility. If one

is needed, we recommend the WD utility WDFMT, located

elsewhere on this BBS. The WDFMT utility will prompt you for

the ACTUAL parameters of the hard drive you are installing.

If you are unsure of your drive's parameters, a file called

DRIVES.LST is available on this TBBS to assist you in most

of the popular (and some unpopular) hard drives.

If you still need assistance ...

WESTERN DIGITAL 24 HOUR TECH SUPPORT ==> 1-800-777-4787