******************************************************************

DESIGN CENTER - 5.4 RELEASE

MICROSOFT WINDOWS EVALUATION VERSION

******************************************************************

This file describes enhancements and modifications made to the 5.4

release of the Design Center.

Also included in this file is information which was not included in the

documentation set provided with the software.

******************************************************************

CONTENTS

******************************************************************

Part 1.  New Features	2

Part 2.  Evaluation Versions	3

Part 3.  General	4

3.1.  Version Interchangeability-Stand Alone Schematics	4

3.2.  New Command Line Option: -i	4

Part 4.  PSpice	4

4.1.  PSpice Command Line Options	4

4.2.  PWL Form for I and V Devices	5

4.3.  PWL Syntax	5

4.4.  PWL Usage Limitations	5

4.5.  New Digital Simulator Operation	6

4.6.  New "O" Device Parameter and Operation	6

4.7.  Digital Worst-Case Timing Analysis Enhancement	7

4.8.  New Example Circuit	7

Part 5.  Probe	8

5.1.  Probe Command Line Options	8

5.2.  Marching Waveforms	8

5.2.1.  Marching Waveforms During Multiple-Run Analyses	8

5.2.2.  Known Problems	9

5.3.  Cut, Copy, and Paste Commands	9

5.3.1.  Using the Clipboard Data	9

5.3.2.  Pasting Objects from the Tools/Label Menu	9

5.4.  Goal Function File	9

Part 6.  Schematics	10

6.1.  Using Schematics Created with Version 5.3 and Earlier	10

6.2.  Schematics Netlister	10

6.3.  New Symbols	10

6.3.1.  STIM Symbols - "source.slb"	10

6.3.2.  PRINT AND PLOT Symbols - "special.slb"	11

6.3.3.  Analog Devices with Visible Pin

Numbers - "analog_p.slb"	11

6.3.4.  Time-Dependent Switches	12

6.4.  Autosave	12

6.5.  Marker Support	12

6.5.1.  Implementation	13

6.6.  Probe Setup Options in Schematics	13

6.6.1.  Restore Last Probe Session	13

6.6.2.  Show All Markers	13

6.6.3.  Show Selected Markers	13

6.6.4.  None	13

6.7.  Changes to the "msim.ini" File	13

6.8.  Accelerator Key Changes	14

6.9.  Font-Related Issues	14

6.9.1.  Using the RASTERFONT=ON/OFF Switch	14

6.9.2.  Using Adobe Type Manager with Schematics	14

Part 7.  Device Equations Option	14

Part 8.  Polaris Signal Integrity	15

Part 9.  Intel IBIS Model Support	16

Part 10.  New Libraries and Devices	20

10.1.  New Library - "AMP.LIB"	20

10.2.  New Devices - "LIN_TECH.LIB" Library	21

10.3.  New Devices in Other Libraries	22

Part 11.  MODTOMDT Utility	22

******************************************************************

Part 1.  New Features

******************************************************************

The following list of new features is applicable to the production

version of the Design Center for Windows.  Some features are not

available for the evaluation version.

DESIGN CENTER

* Solaris support.

* Mentor integration.

* Expanded libraries now with over 5,300 analog and 1,700 digital

devices.

* PC Network license configuration.

* New evaluation version of the Design Center with digital

emphasis including an increase on limits.

SCHEMATICS

* Extended support for external board layout packages: CADStar, P-

CAD, and Tango.

* Stand-alone Schematics package.

* The capability to print a selected area.

* An autosave feature that performs an automatic save of the

schematic file every 10 minutes (by default), if there are any

modifications.  The default time interval can be overwritten by

the user.

* Netlist creation for the entire design from any level of

hierarchy.

* Improved attribute handling features in the Attributes dialog,

such as the ability to exclude non-changeable attributes and

system attributes.

* Marker support of PSpice and Probe output variables for AC

analysis specific elements, such as magnitude, DB (magnitude in

decibels), P (phase), and G (group delay).

* The assignment and re-assignment of user-defined attribute text

layers, which allows the user to group attributes and perform

actions (such as Set Display Level) on the selected group.

* Vertical and horizontal alignment of selected items.  All

selected items can be moved so that their origins, leftmost

endpoints, or centers are vertically or horizontally aligned.

PSPICE

* Improved speed with a 32-bit compiler implementation.

* Lossy transmission line enhancements; the K device has been

extended to allow systems of coupled transmission lines to be

simulated.

* Repetitions in PWL waveform specifications.

* Improved flip-flops/constraint consistency.

PROBE

* Speed improvements.

* Marching Waveforms.

* Scroll bars in Probe for enhancement of viewing zoomed regions.

* Cut, Copy, and Paste commands.

PARTS

* New voltage regulator template.

* Improved MOSFET templates.

* Ability to copy an existing part and use it as a base for a new

part with similar parameter values.

* New extraction algorithm.

* Limitations expansion, for example, Parts accepts depletion mode

JFET characteristics, etc.

POLARIS

* New Signal Integrity analysis tool.

******************************************************************

Part 2.  Evaluation Versions

******************************************************************

The EVALUATION VERSION of the Design Center (release 5.4) has been

enhanced to offer increased capacity for DIGITAL designs, so that you

can experiment with a greater variety of circuits.  Also new for 5.4, is

the ability to model Coupled Transmission Lines.  Hopefully, these will

give you a good feel for the capabilities of the Design Center.  The

EVALUATION VERSION is a full-feature, limited-capacity version of

MicroSim's Design Center product, and is available on the PC under DOS

or Windows, and on the Macintosh.

In general, the maximum device limits are as follows:

- 10 Transistors (any combination of B, M, Q, or J devices)

- 64 Analog Nodes

- 2 Ideal Transmission Lines (Coupled or Uncoupled)

- 2 Lossy Transmission Lines (Coupled or Uncoupled)

(TOTAL of 4 T-Lines maximum, with a maximum of 2 Coupled lines)

- 65 Digital Primitives, excluding STIMulus and PINDLY devices

- Logic output transitions limited to 10000

- "Logic Expression" Primitives limited to 36 I/O Pins

Schematics-related limitations are as follows:

- Maximum of 25 parts on a page

- 1 page only

- A-size page only

- No limit on the number of levels of hierarchy

- 20 new user-defined symbols may be created and used

- No printing from within the Symbol Editor

- Symbols cannot be exported or imported

- The number of symbol library files which can be loaded is limited

to the number of files shipped, plus one

Netlister limitations:

- A maximum of 70 real devices (.net entries) is allowed for PSpice

netlists

- A maximum of 50 symbols, before packaging, may be included in the

parts list for PCB layout netlists

Note, however, that these limitations are maximums for a single class of

devices.  If, for example, you use a combination of transistors and

digital primitives, the allowable number of each will be lower.

******************************************************************

Part 3.  General

******************************************************************

3.1.  Version Interchangeability-Stand Alone Schematics

Warning:  Schematic and/or Symbol Library files which are created and/or

modified using the Stand Alone version of Schematics may not be backward

compatible with previous versions of Schematics.  That is, if you have a

5.3 schematic that you modify using 5.4 Schematics, you may not be able

to read that schematic back into 5.3 Schematics.  Similarly, if you

create symbols using the 5.4 Symbol Editor, these symbols may not be

compatible with previous versions of Schematics (i.e., version 5.3 and

before).

3.2.  New Command Line Option: -i

PSpice, Probe, and Schematics now support a -i command line option which

allows you to specify a configuration file other than "msim.ini."

To use this option, you can change the command line in the program's

icon to be:

<program name>  -i msim54.ini

If no path is specified for the ".ini" file, it is assumed to be in your

Windows directory.  You will need to change the PSPICECMD and PROBECMD

lines in the [MICROSIM] section of "msim54.ini" to also use

-i msim54.ini.

******************************************************************

Part 4.  PSpice

******************************************************************

4.1.  PSpice Command Line Options

The PSpice command line can contain one or more of the following command

line options:

-bf<flush interval>

determines how often (in minutes) PSpice will flush the Probe data file

to disk.  This is useful when a long simulation is left running and the

machine crashes or is rebooted.  In this case, the data file will be

readable up to the last flush. The default is to flush every 10 minutes.

The <flush interval> can be set to a number of minutes between 0 and

1440.  A value of zero means to never flush.

-bn

determines the number of buffers to potentially allocate for the Probe

data file.  Zero buffers means to do all writing directly to disk.

Allocating a large number of buffers can speed up a large simulation,

but will increase memory requirements.  Exceeding physical memory will

either slow down the simulation, or will make it fail.

-bs

determines the size of the individual buffers for writing the Probe data

file.  Using a larger buffer size can reduce execution time, but at the

expense of increasing the memory requirements.  The values for the

buffer files work as follows:

option    -bs0   -bs1   -bs2   -bs3   -bs4   -bs5   -bs6

value      256    512    1024   2048   4096   8192   16384

4.2.  PWL Form for I and V Devices

The general form for "I" and "V" devices is written incorrectly in the

Circuit Analysis Reference Manual.  The correct form is:

For Current ("I" devices):

PWL [TIME_SCALE_FACTOR=<time scale factor>]

+   [VALUE_SCALE_FACTOR=<value scale factor>] (<tn> <in>)*

+   [[REPEAT FOR <n>] (<tn> <in>)* [ENDREPEAT]]*

+   [[REPEAT FOREVER] (<tn> <in>)* [ENDREPEAT]]*

For Voltage ("V" devices):

PWL [TIME_SCALE_FACTOR=<time scale factor>]

+   [VALUE_SCALE_FACTOR=<value scale factor>] (<tn> <vn>)*

+   [[REPEAT FOR <n>] (<tn> <vn>)* [ENDREPEAT]]*

+   [[REPEAT FOREVER] (<tn> <vn>)* [ENDREPEAT]]*

The keywords TIME_SCALE_FACTOR= and/or VALUE_SCALE_FACTOR= must precede

the value of the time scale factor and/or value scale factor.

4.3.  PWL Syntax

Note that parentheses are not required for the time/value data pairs.

Parentheses have been used in the documentation examples for

readability, only.

The example of the PWL data file "triangle.in" included in the Circuit

Analysis Reference Manual discussion inadvertently includes leading "+"

signs.  These should be omitted.

4.4.  PWL Usage Limitations

The Circuit Analysis Reference Manual (Chapter 4, "Analog Devices and

Libraries") states that the PWL form may be described by up to 3995

pairs of data points.  The limitation is now approximately 1000 data

pairs for PC and Macintosh systems, while the number of data points for

the Sun is still virtually unlimited.  (Refer to the discussion of the

PWL form under "V Devices" and "I Devices.")

If you need to use more than 1000 data points, you may still use the

FILE parameter to specify the name of a file which contains the data

points.  Note, however, that the file form does not support repeating

loops.

4.5.  New Digital Simulator Operation

PSpice no longer terminates the simulation when a voltage is out of

range on a digital input pin.  Instead, it now produces a simulation

warning message and uses the state whose voltage range is closest to the

input voltage.  The message will appear in the output file and can be

displayed graphically with the Windows version of Probe.

4.6.  New "O" Device Parameter and Operation

Refer to the discussion of the digital "O" device in Chapter 5, section

5.1.4.2, of the Circuit Analysis Reference Manual.

The following should be added to Table 70, Digital Output Model

Parameters:

Parameter   Description                               Default

---------   -------------------------------------     -------

SXNAME      State applied when the interface node       "?"

voltage falls outside all ranges.

In the same section, replace the paragraph beginning "The state of the

node is determined..." with:

The process of converting the input node voltage to a logic state begins

by first obtaining the difference in voltage between the <interface

node> and the <reference node>.  As you can see, the DOUTPUT model

defines a voltage range, from SxVLO to SxVHI, for each state.  If the

input voltage is within the range defined for the current state, no

state change occurs.  Otherwise, PSpice searches forward through the

model, starting with the current state, to find the next state whose

voltage range contains the input voltage.  This state then becomes the

new state.  When the end of the list (S19) is reached, PSpice wraps

around to S0 and continues.

If the entire model has been searched and no valid voltage range has

been found, PSpice will generate a simulation warning message.  Further,

if the O device is interfacing with the Digital Simulator, and the

SXNAME parameter has not been specified in the model, PSpice will use

the state whose voltage range is closest to the input voltage.

Otherwise PSpice uses SXNAME as the new state.

This "circular" state searching mechanism allows hysteresis to be

modeled directly.  The following model statement models the input

thresholds of a 7400 series TTL Schmitt-trigger input.  Notice the 0.8

volt overlap between the "0" state voltage range and the "1" state

voltage range.

.model DO74_ST doutput (

+       s0name="0"      s0vlo=-1.5      s0vhi=1.7

+       s1name="1"      s1vlo=0.9       s1vhi=7.0

+       )

Starting from the "0" state, a positive-going voltage must cross 1.7

volts to get out of the "0" state's voltage range.  The next state which

contains that voltage is "1".  Once there, a negative-going voltage must

go below 0.9 volts to leave the "1" state's range.  Since no further

states are defined, PSpice wraps around back to state "0", which

contains the new voltage.

4.7.  Digital Worst-Case Timing Analysis Enhancement

A change was made to PSpice to improve accuracy during Digital Worst-

Case Timing analyses of certain circuits.  The change affects the

results of gate primitives, LOGICEXP primitives, and PLD primitives

which have ambiguous rising (R) and falling (F) levels on their inputs.

Gates which are presented with simultaneous opposing R and F levels may

now produce a pulse of the form 0->R->0, or 1->F->1.  For example, a

two-input AND gate with the signals

_____

A  _____/////

_____

B       \\\\\_____

on its inputs may produce

Y  _____/////_____

on its outputs.  This should be interpreted as "a possible single pulse,

no longer than the duration of the R level".  The actual device's output

may or may not change, depending on the transition times of the inputs.

In previous versions of PSpice, gates which were presented with such

inputs would simply produce an unknown (X) pulse on their outputs.

This change corrects a problem in Digital Worst-Case Timing simulations

in certain ambiguity convergence situations.  For example, an ambiguous

clock drives three flip-flops, two of whose outputs drive a gate, which

drives the data input for the third.

____

1--|    |

CLK  -----+-----|>   |----+

|     |____|    |   ___

|      ____     +--|   \        ____

|  0--|    |       |    |------|    |

+-----|>   |-------|___/    +--|>   |---

|     |____|                |  |____|

|                           |

+---------------------------+

In situations where the inputs to the gate are in opposition, PSpice now

correctly realizes that the clock will always precede the arrival of the

data input on the third flip-flop, even though the clock and data have

overlapping ambiguities.

4.8.  New Example Circuit

A new example circuit is now included with PSpice.  It is a digital

frequency comparator, designed using the standard parts contained in the

"7400.lib" library (also new for 5.4).  Supporting components are

included in "frqchkx.lib" and "frqchkx.slb" which are invoked through an

INCLUDE within the schematic.   Stimulus for this example, as well as a

basic description of operating requirements, is contained in

"frqchk.stm."

The schematic diagram for the frequency comparator circuit is named

"frqchkx.sch."  There are also several supporting schematic files

included.  The text of the April 1993 MicroSim Newsletter article

describing the operation of the circuit is included in the file

"frqchk.doc" for your convenience.

One change that has been included in this version of the example is that

the default view of the SDL block now references the "SDLXLE" schematic,

which is a LOGICEXPR-style model used to represent the random gate logic

of the "State Decode Logic" (SDL) block.  It utilizes the "DECODER"

subcircuit (from "frqchkx.lib").  Two other views of SDL exist: the

"gate level" (SDLX) and a PAL implementation (SDLPX).   Use the

Navigate/Push Select Block dialog to choose one of these alternative

views, re-netlist the design, then re-simulate to explore these

representations.

******************************************************************

Part 5.  Probe

******************************************************************

5.1.  Probe Command Line Options

The PSpice command line can contain one or more of the following command

line options:

-bn

determines the number of buffers to potentially allocate for the Probe

data file.  Zero buffers means to do all reading directly from disk.

Allocating a large number of buffers can speed up the reading of a large

Probe data file, but will increase memory requirements.  Exceeding

physical memory will either slow down Probe, or will make it fail.

-bs

determines the size of the individual buffers for reading the Probe data

file.  Using a larger buffer size can reduce execution time, but at the

expense of increasing the memory requirements.  The values for the

buffer files work as follows:

option    -bs0   -bs1   -bs2   -bs3   -bs4   -bs5   -bs6

value      256    512    1024   2048   4096   8192   16384

5.2.  Marching Waveforms

5.2.1.  Marching Waveforms During Multiple-Run Analyses

If  Marching Waveforms (Analysis/Probe Setup/Run Probe During

Simulation) is enabled during a multiple run simulation (i.e., AC sweep,

DC sweep, .STEP, .TEMP, etc.), only the data for the first run will be

displayed in Probe.

To view the family of curves for all runs, close the data file using

File/Close, then reload it using File/Open.  Probe will then prompt for

the sections to be loaded, and the requested traces will be displayed.

5.2.2.  Known Problems

1.   Digital waveforms may march too slowly to be useful.

2.   Disable the DOS "share" command when using the marching waveforms

feature.

5.3.  Cut, Copy, and Paste Commands

5.3.1.  Using the Clipboard Data

The Edit/Cut and Edit/Copy commands are used to place Probe data in the

clipboard.  The following Clipboard Display options are then available

for viewing of the data:

Display/Display Text:  Displays only the Probe trace name

Display/Text:          Displays the data pairs in a proportional font

Display/OEM Text:      Displays the data pairs in a monospaced font

If you want to store the Probe data points as an ASCII file, you should

paste the data from the Clipboard into a text editor of your choice.

(Note that if you try to save the data directly from within the

Clipboard Viewer program some spurious clipboard file data may be

included at the beginning of the file.)

5.3.2.  Pasting Objects from the Tools/Label Menu

The description of this function in the Circuit Analysis Reference

Manual is inaccurate.  The last paragraph on page 288 (section 7.4.1.2)

should read as follows:

"Pasted objects from the Tools/Label Menu (see page 307) will be placed

at the same x and y coordinates that they were at when they were Cut or

Copied.  Once an item from the Label Menu is pasted, it can then be

selected and moved around the plot window."

5.4.  Goal Function File

Included with this release is a file, "msim.gf," which provides various

examples of goal functions for use with Probe.  The file is in ASCII

format and may be viewed with a text editor.

If you already have a goal function file you may wish to append parts of

"msim.gf" to your existing "probe.gf" file.

If you do not have a goal function file, you can rename the file to

"probe.gf" for use with Probe.

******************************************************************

Part 6.  Schematics

******************************************************************

6.1.  Using Schematics Created with Version 5.3 and Earlier

Version 5.4 fixes a problem with moving attributes on flipped and/or

rotated parts.  The attributes would sometimes "bounce" to a position

other than where the mouse was clicked.  However, it does require that

schematics created with version 5.3 (or earlier) of Schematics be run

through a conversion process.  This conversion process will correct the

locations of attributes on flipped/rotated parts so that they display at

the same locations relative to the parts as they did in the version with

which they were created.  If the conversion is not done, some attributes

in your schematic may display/print at different locations than when it

was created.  You will then need to move the attributes manually.

Conversion will be done whenever a schematic written with version 5.3 or

earlier of Schematics is loaded (using File/Open) or pushed into (using

Navigate/Push or double-clicking on the hierarchical part).

You will be prompted first as to whether or not you wish to do the

conversion.  After the conversion is complete, you MUST save the

schematic for the conversion to be permanent.

If you do not wish to convert your schematics (and don't want to be

bothered by the prompt), add

CONVERSION=OFF

to the [SCHEMATICS] section of your "msim.ini" file.

6.2.  Schematics Netlister

The Schematics netlister no longer puts full path names in the .include

statements for the ".als" and ".net" files.  This should make it easier

to copy designs to another directory and resimulate.

6.3.  New Symbols

6.3.1.  STIM Symbols - "source.slb"

Four STIM symbols have been added to the "source.slb" library file:

STIM1, STIM4, STIM8, and STIM16.  STIM1 is for use where only a single

node is being driven.  The other three symbols are used to provide

stimuli to buses that are 4, 8, and 16 bits wide.

Use the DRAW/GET NEW PART menu command, then connect the STIM symbol to

the wire (for STIM1) or the bus (for STIM4, STIM8, and STIM16) to which

the stimulus is to be applied.  Double click on the symbol to edit the

attributes.  Generally, only the FORMAT, TIMESTEP, and COMMAND

attributes need to be modified.

Sixteen COMMAND variables are available,  Normally each command

attribute will contain only one command lime.  It is possible to enter

more than one command line per COMMAND attribute by placing \n+ between

the command lines.  The "n" must be lower case and there are no spaces

between the characters.  (A space may precede or follow the group.)  For

more information on the STIM device, refer to the Stimulus Devices

discussion in the "Digital Devices and Libraries" chapter of the Circuit

Analysis Reference Manual.

6.3.2.  PRINT AND PLOT Symbols - "special.slb"

Six new symbols have been added to "special.slb" in order to expand the

.PRINT and .PLOT options available from Schematics.

VPRINT1 -- generates a print table for the voltage at a single node of

interest.

VPRINT2 -- generates a print table for the voltage differential between

two nodes.

VPLOT1 -- generates a "line printer" plot for the voltage at a single

node of interest.

VPLOT2 -- generates a "line printer" plot for the voltage differential

between two nodes.

IPRINT -- generates a print table for the current through a cut in the

net.

IPLOT -- generates a "line printer" plot for the current through a cut

in the net.

Single terminal devices can be connected directly to the node of

interest.  Voltage differential .PRINT and .PLOT devices should be

connected across the two nodes of interest.  IPRINT and IPLOT symbols

should be inserted into the circuit in series (i.e. break the circuit to

insert the IPRINT or IPLOT), because they are current meters.

Double-click on the symbol.  Select at least one analysis type (TRAN,

AC, or DC depending on the type(s) of analysis being performed).  If AC

is selected, then one or more output formats may be selected (MAGnitude,

PHASE, REAL, IMAGinary, or DB).  To select an analysis type and/or

output format, double-click on the attribute and give it any value.  'Y'

for 'Yes' or the numeral 1 are common values, but anything will work.

Multiple analysis types may be selected provided that those analysis

types are being performed.  If AC is selected, then multiple formats may

be selected.

If no analysis types are selected, the default is transient.  If AC  is

selected, but no output formats are selected, then the default is the

magnitude of the voltage at the connected node.

To generate a print table of the magnitude and phase of the voltage at

the connected node resulting from the AC analysis you might use the

VPRINT1 symbol with the AC, MAG, and PHASE attributes enabled.

For more information on generating print tables and/or "line printer"

plots, see the discussion of .PRINT and .PLOT in the COMMANDS chapter of

the Circuit Analysis Reference Manual.

6.3.3.  Analog Devices with Visible Pin Numbers - "analog_p.slb"

A new symbol library file, "analog_p.slb," has been added for those who

wish to have visible pin numbers on the analog devices R, L, C, R_VAR,

and C_VAR.  Analog_p.slb must be configured by adding it to the list of

configured symbol library files before or in place of the file

"analog.slb."

From within the Schematic Editor, do the following:

1.  Select Configuration/Editor Configuration/Library Settings.

2.  Select (left click) on "analog.slb," enter "analog_p" for File Name,

clear the Package check box and select the Symbol check box, click on

the ADD button, and click on the OK button to approve the changes.

3.  Optionally remove the reference to "analog.slb" by selecting it and

then clicking on the Delete button.

The symbols in "analog_p.slb" will now be used by the Schematic Editor

instead of those in "analog.slb."

6.3.4.  Time-Dependent Switches

Two new symbols and associated models representing time-dependent

switches have been added to "misc.slb" and "misc.lib." The switches are

named Sw_tClose and Sw_tOpen, and they have the following attributes:

Attribute   Description                              Default Value

---------------------------------------------------------------------

tOpen and   Time (Time at which transition begins)   0

tClose

Note: tOpen and tClose apply to the respective model/symbol. They have

the same definition and default value.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

ttran      Transition time (Time required for         1 us

the switch to change states)

Rclosed    Closed state resistance                    0.01 Ohm

Ropen      Open state resistance                      1 Mega-Ohm

Note: ttran, Rclosed, and Ropen are common to both models/symbols.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

6.4.  Autosave

The [SCHEMATICS] section of the "msim.ini" file may contain an entry

specifying the save interval for the Autosave feature.  This entry has

the following form:

AUTOSAVE=<n>

where <n> is the autosave interval in minutes.

Refer to the Schematic Capture User's Guide for more information on the

Autosave feature.

6.5.  Marker Support

Marker support of PSpice and Probe output variables has been added for

AC analysis-specific elements, such as M (magnitude), DB (magnitude in

decibels), P (phase), and G (group delay).

6.5.1.  Implementation

The Schematic Editor Markers menu provides the following new command:

Mark Advanced:

Mark Advanced displays a list of the available symbols in the Marker

Symbol Library file "marker.slb."  This allows you to place markers to

view the magnitude in decibels (dB), phase, and/or group delay.

The markers for dB, phase, group delay, real, and imaginary waveform

characteristics can be used instead of the built-in Probe functions

provided in output variable expressions

6.6.  Probe Setup Options in Schematics

You can now specify whether waveforms will be shown automatically when

Probe is started from Schematics (either automatically after a

simulation, during auto-update, or using the Analysis/Run Probe

command.)

The Analysis/Probe Setup command has been changed to allow you to

specify one of the following:

6.6.1.  Restore Last Probe Session

If this option is enabled, Probe will restore the display using the

display control setup from the previous Probe session.

6.6.2.  Show All Markers

If this option is enabled, traces will be displayed in Probe wherever

markers are located in the schematic. (i.e., performs a Markers/Show

All)

6.6.3.  Show Selected Markers

If this option is enabled, traces will be displayed in Probe wherever

there any selected markers on the current page of the schematic (i.e.,

performs a Markers Show/Selected)

6.6.4.  None

If this option is enabled, no traces will be displayed automatically

when Probe is started.

6.7.  Changes to the "msim.ini" File

The following configuration items have been moved from the [SCHEMATICS]

section to the [MICROSIM] section:

PROBECMD

PSPICECMD

WINDOWSPSPICE

EDITOR

"Marker.slb" should no longer be included in the list of libraries.  If

it is, you will get a warning when you start up Schematics:

"marker libraries should not be in configured list."

This is just a warning; you can continue to use Schematics.

6.8.  Accelerator Key Changes

The accelerators for Cut, Copy, Paste have been changed to <Ctrl X>,

<Ctrl C> , <Ctrl V> to be consistent with Windows 3.1 standards.  The

old accelerators (<Delete>, <Ctrl Insert>, <Ctrl Shift>) are still

supported.

Schematic Editor:

Command     New                Old

--------    --------           -----------

<Ctrl N>    Zoom/Normal        Navigate/Create Page

Symbol Editor:

Command     New                Old

--------    ------------       -----------

<Ctrl N>    Zoom/Normal        Part/New

<Ctrl C>    Edit/Copy          Part/Copy

<Ctrl E>    Package/Edit       <none>

<Alt R>    none               Return to Schematic

6.9.  Font-Related Issues

6.9.1.  Using the RASTERFONT=ON/OFF Switch

If the line RASTERFONT=ON is included in your "msim.ini" file, it causes

the Helvetica font to be used instead of the modern font for display in

Schematics.  This will cause rotated text to appear vertically versus

being rotated on its side.  If your symbols and schematics use rotated

text, setting this item to ON may cause problems with positioning text.

For this reason, we suggest using the RASTERFONT=OFF switch setting.

6.9.2.  Using Adobe Type Manager with Schematics

If you have Adobe Type Manager installed and are using Schematics, you

may encounter problems with how rotated text is displayed.  We recommend

that you check the "ATM.ini" file and comment out the line

Modern=Helvetica

in the [ALIASES] section.  If this is not done, rotated text may be

positioned differently on the display versus on hard copies.  We also do

not recommend using the FONTTYPE=ADOBE setting in your "msim.ini" file

at this time as it has the same problem.

******************************************************************

Part 7.  Device Equations Option

******************************************************************

Use of the Device Equations option is described in Appendix A of the

Circuit Analysis Reference Manual.  Please note the following

corrections the compiler/linker requirements:

The compiler/linker requirement for the Windows version has been changed

from the Microsoft C compiler to the WATCOM C/386 compiler.

Specifically, the information in Appendix A of the Circuit Analysis

Reference Manual should read as follows:

Page 375: A.1, the note should read:

"...you must have the WATCOM C/386, version 9.01.e compiler (for the

Windows version), or the Sun C compiler (for the Sun)."

Page 379, A.3.1 should now read:

"The object and source files necessary to make the Windows version of

PSpice ("wpspice.exe") have been installed in the subdirectory DEVEQU.

A batch file, "devequ.bat," is included to compile and link the program.

"The object code has been compiled with the WATCOM C/386 compiler,

version 9.01.e.  This compiler comes with the necessary Windows SDK

components to compile PSpice.

"Once you have built WPSPICE.EXE, you should rename it to avoid

confusion with the original version.  Then, copy it to the same

directory as the rest of the Design Center programs.  You can then add

it to the Design Center group in the Program Manager so that it can be

invoked from an icon."

******************************************************************

Part 8.  Polaris Signal Integrity

******************************************************************

Translating Alphanumeric Pin Numbers from PCAD Layout Files:

You may encounter problems simulating parasitics extracted from PDIF (P-

CAD) layout files. Problems with regenerating the netlist can occur if

any of the P-CAD library components used in the design have alphanumeric

pin numbers and the corresponding part in the Schematics Symbol Library

has numeric pin numbers. You can tell if this is the case by inspecting

the SPKG constructs contained in the component definitions (COMP_DEF

construct) in your PDIF layout file.

If you find an SPKG construct that contains an Apn construct, you must

ensure that the pin names contained in the P constructs of the pin

definitions for the part (PIN_DEF construct) are numeric (i.e., not

alphanumeric). If the pin names contained in the P constructs are

numeric, you should not have a problem. However, if the pin names in the

P constructs are alphanumeric, and the Apn construct contains only

numeric pin numbers, you can add the following commands to the special

net file, "*.snf."

.PDIF

USEAPN

(The .snf file was created when Schematics initialized the database

translation.) This forces the translator to use the pin numbers

contained in the Apn construct instead of those in the P constructs.

If both the P and Apn constructs contain alphanumeric pin numbers, then

you may have to edit the component definition so that the pin numbers

map correctly between P-CAD and Schematics. Also, if both the P and Apn

constructs contain alphanumeric pin numbers and there is an SP construct

present as well in the SPKG construct, your file contains a syntax

error. This means that most pin number mapping problems for PDIF files

can be resolved by editing the .snf file as described above.

******************************************************************

Part 9.  Intel IBIS Model Support

******************************************************************

A library of Intel's IBIS models will be available in July 1993. They

are Intel's Pentium (TM) 82430 PCIset I/O buffer models.

These highly accurate driver/receiver buffer models include effects such

as switching times, nonlinear transfer characteristics, and package

parasitics.  They allow designers to perform signal-integrity simulation

of I/O interconnects.

An example of an IBIS model follows.

Note that the parameters used in this subcircuit are from IBIS data

published by Intel.

IBIS ESC2 Buffer Model - Worst-Case

*

***   Worst-case:  max ramp time, max package parasitics,

*                   max die Cout, min output currents

*

* The Enable is active high

*

*

*

*              Input  Output  VCC  GND  Enable

.subckt ESC2_W 100    200     300  400  500

+ PARAMS:

+ Cout = 7.2pf           ; die capacitance at output

+ Co_pkg = 2.8pf         ; package bond wire and lead

; capacitance at output

+ Lo_pkg = 15.1nH        ; package bond wire and lead

; inductance at output

+ Ro_pkg = 0.405         ; package bond wire and lead

; resistance at output

+ dVdt_r = {4.75/1.45}   ; output rise time

+ dVdt_f = {4.75/1.61}   ; output fall time

*

*

******* Parameters *******

.PARAM Rtr = { (1/dVdt_r) * 1e5}

.PARAM Rtf = { (1/dVdt_f) * 1e5 }

.PARAM Vinh = 1.56v      ; Input high threshold voltage

.PARAM Vinl = 1.47v      ; Input low threshold voltage

*

*

******* Input and Enable Threshld **********************

Ein_thr_dn 800 0 VALUE = {TABLE(V(100,400),

+ Vinl, 0,

+ Vinl+1e-6, 1)}

*

Ein_thr_up 810 0 VALUE = {TABLE (V(100,400),

+ Vinh-1e-6, 1,

+ Vinh, 0)}

*

Ein_thr 530 0 VALUE = {TABLE (V(500,0),

+ Vinl, 0,

+ Vinh, 1)}

*

Edn 820 0 VALUE = {LIMIT(V(800), 0, V(530))}

*

Eup 830 0 VALUE = {LIMIT(V(810), 0, v(530))}

*

Rdum1 530 0 1G

Rdum2 800 0 1G

Rdum3 810 0 1G

Rdum4 820 0 1G

Rdum5 830 0 1G

*

*

******** Rise & Fall Time *******

*

Gramp_dn 840 400 VALUE={v(840,400) / (abs(v(840,400) * Rtf

+ + V(820,400) * 1e7) + 1e-4) }

Cramp_dn 840 400 0.01pf

*

ERchrg_dn 840 220a VALUE={I(Vchrg_dn) * (abs(v(830) *

+ 1e+9)+1e-4)}

Vchrg_dn 220a 220

cchrg_dn 840 220 1f

*

Gramp_up 300 850 VALUE = {v(300,850) / (abs(v(300,850) * Rtr

+ + V(830,400) * 1e7) + 1e-4)}

Cramp_up 850 300 0.01pf

*

ERchrg_up 850 220b VALUE = {I(Vchrg_up) * (abs(V(820) * 1e9)

+ + 1e-4)}

Vchrg_up 220b 220

cchrg_up 850 220 5f

*

*

******* Output Pulldown *******

Esw_dn 240 0 VALUE = { TABLE(v(840),

+ 4.5-1e-5, 1,

+ 4.5v, 0) }

*

Rout_dn 220 400 1

Gadj_dn 220 400 VALUE = {V(280,220)}

Elim_dn 280 400 VALUE = {LIMIT(v(260), -v(240)*1e3,

+ v(240)*1e3)}

*

*  The following defines the I/V curve of the pulldown

*  structure:

Epwl_dn 260 0 TABLE {V(220,840)}

+ -5.0v, -110m

+ -4.0v, -107m

+ -3.0v, -103m

+ -2.0v, -91m

+ -1.0v, -57m

+ 0.0v, 0m

+ 0.5v, 31m

+ 1.0v, 57m

+ 1.5v, 76m

+ 2.0v, 91m

+ 2.5v, 100m

+ 3.0v, 103m

+ 3.5v, 105m

+ 4.0v, 107m

+ 4.5v, 109m

+ 5.0v, 111m

+ 10.0v, 120m

Rdum6 240 0 1G

Rdum7 260 0 1G

Rdum8 280 400 1G

*

******* Output clamp to GND *******

**  The following defines the I/V curve of the clamping

**  diode connected to GND:

Ggnd_out 220 400 TABLE {V(220,400)}

+ -5.0v, -1725m

+ -1.0v, -110m

+ -0.9v, -70m

+ -0.8v, -35m

+ -0.7v, -5m

+ -0.6v, 0u

+ -0.5v, 0u

+ 0.0v, 0u

+ 10.0v, 0u

*

******* Output pullup *******

*****

Esw_up 230 0 VALUE = {TABLE(V(850),

+ 0.5v, 0,

+ 0.5+1e-5, 1 )}

Rout_up 300 220 1

Gadj_up 300 220 VALUE={V(220,270)}

Elim_up 300 270 VALUE={LIMIT(V(0,250), -V(230)*1e3,

+ V(230)*1e3)}

*

*  The following defines the I/V curve of the pullup

*  structure:

Epwl_up 250 0 TABLE={V(850,220)}

+ -5.0v, 57m

+ -4.0v, 54m

+ -3.0v, 50m

+ -2.0v, 40m

+ -1.0v, 23m

+ 0.0v, 0m

+ 0.5v, -13m

+ 1.0v, -23m

+ 1.5v, -33m

+ 2.5v, -46m

+ 3.0v, -50m

+ 3.5v, -52m

+ 4.5v, -55m

+ 5.0v, -57m

+ 10.0v, -66m

Rdum9 230 0 1G

Rdum10 250 0 1G

Rdum11 270 300 1G

*

******* Output clamp to VCC

*  The following defines the I/V curve of the clamping diode

*  connected to VCC:

GVcc_out 220 300 TABLE {V(300,220)}

+ -5.0v, 970m

+ -1.0v, 74m

+ -0.9v, 52m

+ -0.8v, 30m

+ -0.7v, 4m

+ -0.6v, 0m

+ -0.5v, 0m

+ -0.4v, 0m

+ 0v, 0m

*

******* Output Package *******

**

Cout_pkg 200 400 {Co_pkg}

Rout_pkg 200 210 {Ro_pkg}

Lout_pkg 210 220 {Lo_pkg}

Rout_snb 210 220 100

Cout_die 220 400 {Cout}

******* End of Subcircuit *******

****

.ENDS ESC2_W

These models are not part of the libraries shipped with the 5.4 release.

If you currently have version 5.4 of the Design Center and would like to

receive these models, please send your business card, with your program

ID number written on the back, to:

MicroSim Corporation

Pre-release IBIS Models

20 Fairbanks

Irvine, CA 92718

or call 1-800-245-3022 with that information.  The above example model

is also available on MicroSim's BBS.

******************************************************************

Part 10.  New Libraries and Devices

******************************************************************

This section lists the new libraries and devices added to the 5.4

release which are not documented in the documentation set.

10.1.  New Library - "AMP.LIB"

This library provides the following connector models:

- C -

CHHDSM/AMP    CHHDSAL/AMP    CHS1B/AMP      CHS1SB/AMP

CHHDSA/AMP    CHHDSBL/AMP    CHS1C/AMP      CHS1SC/AMP

CHHDSB/AMP    CHHDSCL/AMP    CHS1D/AMP      CHS1SD/AMP

CHHDSC/AMP    CHHDSDL/AMP    CHS1ML/AMP     CHS1SML/AMP

CHHDSD/AMP    CHHDSEL/AMP    CHS1AL/AMP     CHS1SAL/AMP

CHHDSE/AMP    CHHDSFL/AMP    CHS1BL/AMP     CHS1SBL/AMP

CHHDSF/AMP    CHHDSGL/AMP    CHS1CL/AMP     CHS1SCL/AMP

CHHDSG/AMP    CHHDSHL/AMP    CHS1DL/AMP     CHS1SDL/AMP

CHHDSH/AMP    CHS1M/AMP      CHS1SM/AMP

CHHDSML/AMP   CHS1A/AMP      CHS1SA/AMP

- E -

EEC4M/AMP     EEC5B/AMP      EISAAL/AMP     ETCBL/AMP

EEC4A/AMP     EEC5C/AMP      EISAEL/AMP     ETCCL/AMP

EEC4B/AMP     EEC5ML/AMP     EISABL/AMP

EEC4C/AMP     EEC5AL/AMP     EISAFL/AMP

EEC4ML/AMP    EEC5BL/AMP     ETCM/AMP

EEC4AL/AMP    EEC5CL/AMP     ETCA/AMP

EEC4BL/AMP    EISAA/AMP      ETCB/AMP

EEC4CL/AMP    EISAE/AMP      ETCC/AMP

EEC5M/AMP     EISAB/AMP      ETCML/AMP

EEC5A/AMP     EISAF/AMP      ETCAL/AMP

- F -

FB2MM/AMP     FB2MC/AMP      FB2MAL/AMP     FB2MDL/AMP

FB2MA/AMP     FB2MD/AMP      FB2MBL/AMP

FB2MB/AMP     FB2MML/AMP     FB2MCL/AMP

- H -

HDI3M/AMP     HDI4M/AMP      HDI4SAL/AMP    HM7M/AMP

HDI3A/AMP     HDI4A/AMP      HDI4SBL/AMP    HM7A/AMP

HDI3B/AMP     HDI4B/AMP      HDI4SCL/AMP    HM7B/AMP

HDI3C/AMP     HDI4C/AMP      HDI4SDL/AMP    HM7C/AMP

HDI3ML/AMP    HDI4D/AMP      HM5M/AMP       HM7D/AMP

HDI3AL/AMP    HDI4ML/AMP     HM5A/AMP       HM7E/AMP

HDI3BL/AMP    HDI4AL/AMP     HM5B/AMP       HM7ML/AMP

HDI3CL/AMP    HDI4BL/AMP     HM5C/AMP       HM7AL/AMP

HDI3SM/AMP    HDI4CL/AMP     HM5D/AMP       HM7BL/AMP

HDI3SA/AMP    HDI4DL/AMP     HM5E/AMP       HM7CL/AMP

HDI3SB/AMP    HDI4SM/AMP     HM5ML/AMP      HM7DL/AMP

HDI3SC/AMP    HDI4SA/AMP     HM5AL/AMP      HM7EL/AMP

HDI3SML/AMP   HDI4SB/AMP     HM5BL/AMP      HSCEM/AMP

HDI3SAL/AMP   HDI4SC/AMP     HM5CL/AMP      HSCEML/AMP

HDI3SBL/AMP   HDI4SD/AMP     HM5DL/AMP

HDI3SCL/AMP   HDI4SML/AMP    HM5EL/AMP

- I -

ISAM/AMP

ISAML/AMP

- L-

LPG27A/AMP    LPG27AL/AMP

LPG27B/AMP    LPG27BL/AMP

LPG27C/AMP    LPG27CL/AMP

- M -

MD5S25M/AMP   MI25SML/AMP    MSRAAL/AMP     MS73M/AMP

MD5S25ML/AMP  MI50M/AMP      MSRABL/AMP     MS73A/AMP

MD5S32M/AMP   MI50ML/AMP     MSRACL/AMP     MS73B/AMP

MD5S32ML/AMP  MI50SM/AMP     MSRADL/AMP     MS73C/AMP

MEPGA/AMP     MI50SML/AMP    MS43A/AMP      MS73D/AMP

MEPGB/AMP     MODU39M/AMP    MS43B/AMP      MS73ML/AMP

MEPGC/AMP     MODU39ML/AMP   MS43C/AMP      MS73AL/AMP

MEPGAL/AMP    MSRAM/AMP      MS43D/AMP      MS73BL/AMP

MEPGBL/AMP    MSRAA/AMP      MS43ML/AMP     MS73CL/AMP

MEPGCL/AMP    MSRAB/AMP      MS43AL/AMP     MS73DL/AMP

MI25M/AMP     MSRAC/AMP      MS43BL/AMP

MI25ML/AMP    MSRAD/AMP      MS43CL/AMP

MI25SM/AMP    MSRAML/AMP     MS43DL/AMP

- S -

SES5M/AMP

SES5ML/AMP

- T -

TBC4M/AMP     TBC5B/AMP      TBC6B/AMP      TPG35A/AMP

TBC4A/AMP     TBC5C/AMP      TBC6C/AMP      TPG35B/AMP

TBC4B/AMP     TBC5D/AMP      TBC6D/AMP      TPG35C/AMP

TBC4C/AMP     TBC5E/AMP      TBC6E/AMP      TPG35AL/AMP

TBC4D/AMP     TBC5ML/AMP     TBC6F/AMP      TPG35BL/AMP

TBC4ML/AMP    TBC5AL/AMP     TBC6ML/AMP     TPG35CL/AMP

TBC4AL/AMP    TBC5BL/AMP     TBC6AL/AMP     TPI35A/AMP

TBC4BL/AMP    TBC5CL/AMP     TBC6BL/AMP     TPI35B/AMP

TBC4CL/AMP    TBC5DL/AMP     TBC6CL/AMP     TPI35C/AMP

TBC4DL/AMP    TBC5EL/AMP     TBC6DL/AMP     TPI35AL/AMP

TBC5M/AMP     TBC6M/AMP      TBC6EL/AMP     TPI35BL/AMP

TBC5A/AMP     TBC6A/AMP      TBC6FL/AMP     TPI35CL/AMP

- Z -

ZPSLM/AMP     ZPSLML/AMP

ZPSLA/AMP     ZPSLAL/AMP

ZPSLB/AMP     ZPSLBL/AMP

ZPSLC/AMP     ZPSLCL/AMP

ZPSLD/AMP     ZPSLDL/AMP

10.2.  New Devices - "LIN_TECH.LIB" Library

This library provides the following opamp models:

LM10C/LT           LT1200/LT           LT1222/LT

LT1001S8/LT        LT1201/LT           LT1223/LT

LT1007CS/LT        LT1202/LT           LT1224/LT

LT1028CS/LT        LT1208/LT           LT1225/LT

LT1037CS/LT        LT1209/LT           LT1226/LT

LT1187/LT          LT1211/LT           LT1227/LT

LT1189/LT          LT1212/LT           LT1228/LT

LT1190/LT          LT1213/LT           LT1229/LT

LT1191/LT          LT1214/LT           LT1230/LT

LT1192/LT          LT1215/LT           LT1252/LT

LT1193/LT          LT1216/LT           LT1253/LT

LT1194/LT          LT1217/LT           LT1254/LT

LT1195/LT          LT1221/LT           LT318A/LT

10.3.  New Devices in Other Libraries

The following libraries also contain new devices.  Refer to the appendix

in the Circuit Analysis Reference Manual for specific device names.

AMP.LIB *          HARRIS.LIB

ANLG_DEV.LIB       LINEAR.LIB

APEX.LIB           LIN_TECH.LIB *

BURR_BRN.LIB       NAT_SEMI.LIB

COMLINR.LIB        POLYFET.LIB

ELANTEC.LIB

(*)Additional parts not listed in the Circuit Analysis Reference Manual

are listed in previous sections of this file.

******************************************************************

Part 11.  MODTOMDT Utility

******************************************************************

MODTOMDT is a utility program to convert PSpice model files or library

files containing PSpice models to MDT files in Parts format.  The MDT

files generated will be compatible with other MDT files generated by

Parts.

Currently, the following device types are supported:

DIODE        JFET

BJT          MOSFET (LEVEL 3)

The following lists features and usage of this utility:

1.  Allow users to load a model file into Parts and see the performance

curves of the model.

2.  Allow users to perform a "what-if" analysis when they change the

model parameters in Parts.

3.  Allow our users to modify/update the model by entering screen data

into the Parts program and re-fit some model parameters

Usage:      modtomdt [options] <infile>

An input file must be provided.  This could be a single model file, or

it could be a .LIB file that contains many .MODEL statements.

Command line options:

-h   provide a summary of usage

-n   force all answers to program queries to 'No'

-y   force all answers to program queries to 'Yes'

Each .MODEL statement in a file will result in an individual MDT file.

If the output MDT file already exists, the user is prompted to confirm

overwriting the existing file.

An example:

A user wants to use a device, "A," for PSpice simulation, but cannot

find the model in the library.  However, a similar device, "B," is in

the library.

Here is the procedure for creating the A.MDT file using Parts and the B

device:

1.   Copy the model for device B from the appropriate library file to a

file <infile>.  (The model begins with a .MODEL statement.)

2.   Run MODTOMDT <infile> to create B.MDT.

3.   Rename B.MDT to A.MDT.

4.   Run Parts, and load A.MDT.

5.   Modify/Add data using the Parts program, then generate the file

A.MOD.