SF-LAN.DOC

Running SPITFIRE v3.3 on a LANtastic Network

SPITFIRE - Version 3.3

Copyright (C) by Mike Woltz, 1987-1993

Buffalo Creek Software

RUNNING SPITFIRE 3.3 ON A LANTASTIC NETWORK

-------------------------------------------

Running SPITFIRE v3.3 on a Local Area Network allows maximum speed as

well as capability of all multi-node strategies.  While hardware that

is required is considerable, LAN systems allow for performance not

possible using a multitasking system.

Information discussed in this section while specific to the LANtastic

Network is similar to other LAN systems such as Novell, etc.

Memory Requirements:

-------------------

LANtastic requires less memory than any LAN currently on the market.

On a file SERVER (the CPU which contains hard disk(s) in use by the

Network), three programs must be loaded which require about 40K.

On a WORKSTATION (a CPU that may have no disks at all), two programs

are used and require about 14K.  In addition, the DOS SHARE program

must be loaded on all WORKSTATIONS.  If the machine is an 80386

which has high memory available, any or all of these drivers can be

loaded there, freeing up conventional memory.

Hardware Requirements:

---------------------

A Local Area Network contains at least one SERVER and a WORKSTATION.

Some just contain Servers, meaning that they all share hard drives.

While fast CPUs such as 80286 or 80386 machines yield superior

performance, 8086 and 8088 based compatibles can be used.  In

addition, Diskless Workstations which boot from ROM then connect to

the Network can be utilized.

Each machine must have a Network card installed.  The 2MB/sec cards

made by LANtastic or Ethernet Compatible 10MB/sec cards can be used.

One 8 or 16 bit slot per CPU is required where appropriate.  Cables

and terminating resistors are supplied with the Network Startup Kits.

Starting It All Up:

-------------------

LANtastic provides an excellent software installation program which

is executed for each Server and Workstation on the Network.  After

installing all necessary hardware, the software must be installed.

If any nodes utilize the SPITFIRE printer option, the LANtastic

software must be configured accordingly so the printer can be

shared across the Network.  Setting redirector buffers to anything

other than the default may yield unpredictable printouts.  If the

default number of buffers cannot be used, experimentation may be

necessary to determine the correct number of buffers for the LAN.

Set up a separate SPITFIRE directory for each machine, and edit

each CONFIG.SYS and AUTOEXEC.BAT to both load all the LANtastic

programs used and to automatically boot SPITFIRE.  Set CONFIG.SYS

FILES= to the number of nodes multiplied by 25 files per node on

Servers.  For example, if the Network has one Server and three

Workstations associated with it, and all have SPITFIRE nodes set

up on them, set FILES=100 on the Server and FILES=25 on each

associated Workstation.  These numbers might need to be increased

further when used with net-mail systems.

Workstations that boot when the associated Server is still

booting or not available will wait until the Server comes online.

This will occasionally yield the familiar "gleep" sound known by

anyone who has been near a LANtastic Network!  Once the Server

finishes its boot routine, it will bring up its SPITFIRE Node and

all others on associated Workstations will follow.

In the event that any machines on the Network use OS/2 instead of

DOS, it is important to remember that DOS does NOT recognize any

drive or partition that has been formatted with the OS/2 High

Performance File System (HPFS).  It is therefore recommended that

any HPFS partitions be the final drive on the system so that the

LAN will not have to "skip over" a drive that is not available.

Multitasking On A LAN:

----------------------

Each machine, whether a Server or a Workstation, can have at least

one node installed.  In order to install other nodes on the Network,

additional machines can be added.  At this time, DESQview can ONLY

be used on WORKSTATIONS! Windows 3.X can be used on both Servers

and Workstations providing LANtastic NOS of 4.0 or greater is in use.

Running DESQview on a LANtastic Server will cause the system to

crash.  DESQview 386 and Windows 3.0 (in 386 Enhanced Mode) can be

successfully run on a Workstation yielding as many nodes as memory

and available serial ports will allow.  Refer to the appropriate

documentation (SF-WIN.DOC or SF-DESQ.DOC) for running SPITFIRE under

DESQview and Windows for further information.