Taper v1.01                                                 (Release 19-06-1997)

Written by M. van der Heide, Martin Kopanske and Tomaz Kac

Copyright 1997 ThunderWare Research Center

This program may be used for maintenance purposes of your ZX-Spectrum tapes and

Spectrum emulator tape files.

It can be used as a virtual tape unit in conjunction with a real ZX-Spectrum.

Supported tape file formats are .TAP, .LTP, .ZXS (readonly), .VOC and .TZX.

0.0 STANDARD DISCLAIMER

-----------------------

This program is provided "as is", without any warranties, nor an indication

that it will fit the purpose.

TAPER has been tested on a lot of systems, including '286, '386, '486,

'486DLC and Pentium machines (even emulators), with ISA, VLB and PCI causing

no problems. If you want to use this program, though, you do so at your own

risk. This means that it is extremely unlikely to damage your system, but if

the unthoughtful event DOES happen, you will be on your own.

ThunderWare Research Center can and will accept no claims whatsoever.

This program is neither public domain nor shareware: it's freeware.

This means that anyone is allowed to distribute and use the program, but all

copyrights are still owned by ThunderWare Research Center.

You can therefor not request any money from anyone for copying this program,

apart from the expenses made for the media onto which it has been copied.

Remember you can always freely obtain the latest version from the Spectrum

page at:

http://www.void.demon.nl/spectrum.html

Or you can try:

ftp://ftp.nvg.unit.no/pub/sinclair/utils/pc

0.1 BETA DISCLAIMER

-------------------

For beta versions of TAPER, the same legal restrictions apply.

As such a version is uncertain to be stable, using one is at your own risk.

We try to make beta versions that are harmless to the system even when failing,

but chances of failures are more likely than in normal versions.

0.2 INDEX

---------

1.   Requirements

2.   Important Notes

3.   Sytax

4.   The Screen

5.   Fill-In Fields

6.   Working With Tape Entries

6.1. Flag/Parity Behavior

7.1. The Configuration

7.2. The Configuration File

8.   The Menu Strip

9.   Tape Catalogs

10.   Using a Custom Palette

11.   Using a Soundcard

12.   Troubleshooting

13.   The Credits

14.   How to Contact Us

1. REQUIREMENTS

---------------

In order to be able to use this program you will need the following hardware:

- A 80286 or better processor,

- A (Super)VGA card with at least 256Kb,

- A mouse with a fully Microsoft compatible driver

(Microsoft compatibility version 6.26 or up),

- At least 2Mb of memory,

- At least 500Kb of free conventional memory, or 450Kb if (enough) upper

memory is available,

- A SoundBlaster (compatible) soundcard if you want to playback tapes to a real

ZX-Spectrum.

And the following software:

- (MS)DOS 5.0 or better,

- An XMS driver (e.g. HIMEM or QEMM) with XMS version 2.0 or higher,

See HISTORYT.DOC for the latest changes.

See VOCDECOD.DOC for information about VOC decoding.

2. IMPORTANT NOTES

------------------

If you want to use options that are not yet in your configuration file,

simply start TAPER, choose `Config' and `Save'. This writes the configuration

file back, but with the new options added (at their default values).

1. If a fatal error occurs, you will be asked if an attempt should be made to

rescue the database (if it had been changed). If a successful attempt was

made, the database will be in the file `xxxxTAPE.TZX', where xxxx is

LEFT for the left window and RGHT for the right window.

2. If the following fatal error occurs after writing back a changed database:

`Could not install new tape', or `Could not remove old tape', the version

just written will have an extension `.TMP' (in the same directory as the

old database).

You must rename this file yourself before restarting.

3. Although this program runs fine in a DOS-box from Windows, this is not the

recommended setup, as timing routines don't like Windows that much

(and neither should you)! Also see the Troubleshooting section on Windows.

Make sure you allow use of the upper memory area, as this frees a large

amount of precious conventional memory.

4. Unlike a certain well-known Microsoft product, this GUI allows dragging and

resizing while the window is being updated at the same time (try dragging

the percentage requester). And no, it wasn't hard to implement...

5. Some options need a temporary directory to be defined.

In DOS, you define this with the command (in your AUTOEXEC.BAT)

SET TMP=<path>

Windows users will probably have a `TEMP' setting instead.

TAPER will look for `TMP' first. If this is not set it will look for `TEMP'.

If neither one is set, it will use the TAPER directory.

If you start TAPER from a floppy or CD-ROM, you MUST have one of `TMP' or

`TEMP'!

If you want some real speed, create a RAM disk and make `TMP' point to it.

3. SYNTAX

---------

To see what syntax is available, use TAPER -?

To start TAPER, use the syntax

TAPER [-w] [-u] [<tapefile1>] [<tapefile2>]

Option `-w' is used to disable MS Windows `multitasking' if it is detected.

It will be enabled again once you leave TAPER. This should be used with caution.

This option works on Windows 3.x in enhanced mode and on Windows95.

Of course, programs run from the shell will still detect that Windows is running

and act accordingly, but timing will be more accurate.

This option should only be used on slow machines.

Option `-u' is used to disable use of upper memory.

'<tapefile1>' will be loaded into the source window (left).

'<tapefile2>' will be loaded into the destination window (right).

4. THE SCREEN

-------------

The screen is set up in a 640x480x16 resolution and contains two windows.

The left window holds the source tape, the right window the destination tape.

At the top of these windows is a `128K' button. This is used only to remap

characters according to the 128K Spectrum when printing a BASIC listing.

At the same height is a Size box. This size is the total amount of virtual

memory that the currently loaded tape takes.

At the bottom of these windows is the name of the current tape printed,

together with a checkmark if the tape has been changed.

Each window is formatted as follows:

- entry number (starts at 0),

- '*' for a custom loading block, or blank otherwise,

- entry description.

Between these two windows you will find the following buttons:

Config  - Configure the application.

Shell   - Shell out to DOS. Notice that the application consumes only 2Kb of

conventional memory :)

WARNING: you must not start TSR programs from within the shell, or the

application will not be able to swap back in!

If MS Windows multitasking has been disabled, it is still disabled

while you're in a shell.

Quit    - Stop.

Help    - View this file.

About   - Shows an about box.

SysInfo - Shows some system information.

The buttons can be selected in two ways:

1. By clicking on them,

2. By pressing [ALT] + the underlined character.

You can make a snapshot (GIF image) of the full screen at any time by pressing

[CTRL][Print Screen]. In each session, the first screen dump will be called

SDUMP001.GIF, the next SDUMP002.GIF etc.

5. FILL-IN FIELDS

-----------------

A lot of fields in requesters can be edited. Click on it to get a cursor.

The keys are:

[CURSOR LEFT]  - One char left;

[CURSOR RIGHT] - One char right;

[HOME]         - Go to the start of the box;

[END]          - Go to the end of the box;

[BACKSPACE]    - Delete the character before the cursor;

[DEL]          - Delete the character under the cursor;

[INS]          - Toggle insert/overstrike mode (the startup mode is set in the

configuration window);

[ENTER]        - Accept the input and elect next button;

[TAB]          - Accept the input and elect next button;

[SHIFT][TAB]   - Accept the input and elect previous button;

[ESC]          - Restore and stop editing;

[CTRL] and one of:

[CURSOR LEFT]  - One word left;

[CURSOR RIGHT] - One word right;

[HOME]         - Go to the start of the input field;

[END]          - Go to the end of the input field;

[DEL]          - Clear the entire input field.

B              - Clear start of input field up to the cursor;

E              - Clear from the cursor to the end of the input field;

W              - Clear from the cursor to the start of the next word;

Any ASCII character inserts the character.

Some fields only accept numerical characters.

If you start editing, overstrike or insert mode is selected depending on the

configuration.

6. WORKING WITH TAPE ENTRIES

----------------------------

Almost every tape entry can be selected. The current entry is inverted.

To select another entry, either move the slider and click on it with the mouse

or use the keys:

[CURSOR UP]   - Move one entry up;

[CURSOR DOWN] - Move one entry down;

[PAGE UP]     - Move a page up;

[PAGE DOWN]   - Move a page down;

[INS]         - Choose the entry;

[CTRL][HOME]  - Move to the first entry;

[CTRL][END]   - Move to the last entry;

[CTRL][INS]   - Toggle the `selected' status of the entry.

To toggle the `selected' status of an entry with the mouse, hold down [CTRL] or

the middle mouse button (if you have a 3-button mouse) while clicking on the

game name.

You can toggle the `selected' status of an entire range of entries at once (with

the mouse only!) by selecting the entry at the beginning of the range and then

selecting the last entry in the range while pressing [SHIFT].

`Selected' entries are printed with a cyan background.

The slider can be used as follows:

- Pick up the slider and drag it to the position you want;

- Hold down [CTRL] or the middle mouse button and click somewhere on the

slider bar to set the slider to that position directly;

- Click above the slider to go up a page (repeated while pressed);

- Click below the slider to go down a page (repeated while pressed);

- or use the arrow gadgets to go up/down a line (repeated while pressed);

Entries that have an error in it are printed in red.

Entries that loose data when saved (unless saved as VOC) are printed in bright

red.

Descriptive entries are printed in blue.

When you double-click an entry, you get a window showing details about it.

You can drag a tape entry from one window and drop it in either the same or the

other window.

If you drag a selected entry, all selected entries will be copied at once.

If you want to move the entry rather than copy, hold down [ALT] when you start

dragging it.

6.1. FLAG/PARITY BEHAVIOR

-------------------------

In order to display tape blocks correctly, the following simple rules are

applied to flag and parity bytes:

- Entry types Normal Speed (0x10) and Turbo Speed (0x11) always have both a

flag byte and a parity byte, unless the total block is less than 2 bytes;

- Entry type Pure Data (0x14) doesn't have a flag nor a parity byte. Instead,

both bytes are kept into the data block, but are displayed between brackets

when viewing the entry info (the parity byte field has 2 values displayed -

the last byte in the block and the calculated 'parity').

If the last byte is incomplete (less than 8 bits), the one-last byte is used

as parity byte.

7.1. THE CONFIGURATION

----------------------

The configuration window holds the following items:

Remap Bright Black  : Should the ULA bug be matched and should bright black be

displayed as dark black?

Emulate Flash       : If turned on, flash is emulated when drawing Spectrum

screens.

Slow Flash          : Normally, flashing is done every 16/50th of a second

(as the Spectrum itself does). If you have a slower

computer or video card, this may result in loss of

responsiveness. In this case, you should turn this

option on, in which case flashing is reduced to a

third of the time (just under a second).

Both these options can be toggled from the menu strip.

Confirm Quit        : User must confirm quit?

Confirm Delete      : User must confirm delete?

Confirm Overwrite   : If turned on, TAPER will ask for confirmation when a file

is about to be overwritten (except the tape itself).

Follow Slider       : If turned on, a combo window will be updated while you

move the slider bar. If turned off, only the slider bar

will be updated while moving; the result will be printed

when you release the mouse button.

This option should be turned off for slower computers

only.

Edit Insert         : Sets the initial edit mode for startup of edit buttons.

Checked means insert mode, cleared means overstrike mode.

Always Backup       : If this option is turned on (the default), TAPER will make

a backup of the tape the first time that a new version is

to be written. This file will have extension `.BAK'

This option can be temporarily overridden from the menu.

Disable Sounds      : Disable all speaker sounds from TAPER.

Trace Sound Blocks  : If turned on, TAPER will constantly show the currently

played block when the `Play' menu option is used.

This option can be temporarily overridden from the menu.

It should be turned off for slower computers only.

Blanker Installed   : Use the blanker?

Blanker Time-Out    : Number of seconds of non-keyboard/mouse activity before

the blanker activates.

Button `Save' will save the current configuration, `Reload' will load the

configuration back from disk.

You must explicitely press the `Use' button (or the close gadget or the [ESC]

key) to leave the configuration window.

7.2. THE CONFIGURATION FILE

---------------------------

The file TAPER.INI holds the above configuration. Other values are:

DragOutlineOnly     : If turned on, will update only the window's outline

while a window is being dragged. This should only be

turned off if you have an extremely fast processor and

video card (as most windows reside in XMS)

ResizeOutlineOnly   : If turned on, will update only the window's outline

while a window is being resized. This should only be

turned off if you have an extremely fast processor and

video card (as most windows reside in XMS)

PreLoadResources    : Load all font files at startup when set. This will save

some floppy swapping when running TAPER from floppies and

will prevent TAPER from getting out of memory a bit.

DirectVideoControl  : Do not use the RIL if one has been detected. This will

speed up all screen functions, but can cause a messed up

screen if your video card cannot handle it.

NextFloppyDiskId    : This is the next floppy number to be used. You should

NOT edit this field by hand!

UseGeniusMouseHiding: If turned on, will use the old-style Genius mouse hiding

function. Turn it on only when you have a Genius mouse

driver with version 7.02 or below.

NoLocalMouseHiding  : If turned on, will always fully hide the mouse when a

request is made to hide in a specific range.

This should only be used when you have a really weird

mouse driver (e.g. in Windows NT/95) that messes up the

screen. It will slow down the GUI considerably.

ForceMouseRelease   : To be used only if you have a weird mouse that seems to

be lost (doesn't work in a next application) if you quit.

SwapMouseButtons    : If turned on, will swap the left and right mouse buttons.

SnapMouseDefault    : Turn it on if you want the mouse to move to the default

button if a window/requester is drawn or if an edit

button is finished with the [ENTER] key (Default is off).

DoubleClickInterval : The maximum time elapse in milliseconds between two mouse

clicks to be recognised as a double-click.

InitialContPushDelay: The initial delay in milliseconds when keeping a

continuous push button pressed (e.g. the arrow buttons on

sliders).

UseCustomPalette    : see `Using a Custom Palette' below.

CustomColorXX       : see `Using a Custom Palette' below.

The above parameters must be filled in by hand.

This file can be edited with any ASCII editor, but is also rewritten if you

change the configuration.

8. THE MENU STRIP

-----------------

TAPER has a menu strip a la Amiga. For users unfamiliar with the Amiga, I'll

explain how it works:

- To use the menu strip, press the right mouse button. You'll see the strip

at the top line of the screen;

- While keeping the right button pressed, move it to one of the words in it.

A roll-down menu will appear;

- While still keeping the right mouse button pressed, move the mouse over the

menu entry you wish to activate. Entries with an `>>' at the end have a

submenu attached;

- When the mouse highlights the entry you want, release the right mouse button;

- If you don't want one of the options, move the mouse somewhere in the void

and release the right mouse button there.

- Some menu entries have key shortcuts. These are shown at the end of a line.

The menu strip has the following entries:

1/3 x Window (x = '<' or '>')

- New              : Remove the current tape from memory and start with

an empty one. You will be asked to save first if you

changed anything;

Notice that the tape name will be `(new tape)'.

When saving a new tape, you should use the menu

option `Save As...' to give it a proper name.

- Open ...         : Remove the current tape from memory and open a new

one. You will be asked to save first if you changed

anything;

- Save As ...      : Save the tape to a new file;

- Save             : Save the tape;

- Import tape      : Join another tape in memory. The new tape will be

simply added.

Afterward, the selection bar will be on the first

imported line.

- Backup tape      : If checked, will create a backup of the `old' tape

before writing a new one (to temporarily override the

`Always Backup' option from the configuration window).

- Auto-decrypt     : SpeedLock 2, 3 and 4 blocks are encrypted. Most of them

have a fixed encryption scheme. If the tape file

contains a 'Group Start' entry with a group name

starting with 'SpeedLock 2', 'SpeedLock 3' or

'SpeedLock 4' and this option is turned on, the blocks

are automatically decrypted when using a 'View As' menu

option.

- Quit             : Guess...

2/4 x Entry (x = '<' or '>')

- Deselect all      : Make all games unselected;

- Invert selected   : Invert the selected games - selected games are now

deselected and vice versa;

- Insert ...        : Insert a tape entry manually;

- Delete            : You will see a submenu:

- Entry           : Delete the current entry;

- Selected        : Delete all selected entries at once;

- Duplicate entry   : Duplicate the currently selected entry;

- Play              : You will see a submenu:

- Entry           : Play the current entry through the soundcard;

- Selected        : Play all selected entries through the soundcard;

- From entry      : Play from the current entry up to the end of the tape

through the soundcard;

- All tape        : Play the entire tape through the soundcard;

- Save raw data     : Save the data of the currently selected tape entry to

a seperate file, excluding flag/parity bytes. This can

be used to create .SCR files for example.

- View as           : You will see a submenu:

- Memory dump     : View in both hexadecimal and ASCII;

- Header          : View as if the entry is a header (first 17 bytes)

- BASIC listing   : View as if the entry is a 'Program:' data block;

The listing is shown in a seperate window, together

with the button 'Drop Colors' (Remove the colors from

the output);

- Screen image    : View as if the entry is a loading screen;

- View entry info   : View detailed info on the current entry (the same as

double-clicking the entry);

5. System

- Disable sounds    : Turn all sounds from TAPER on/off. Has the same purpose

as the command line option `-q'.

- Emulate flash     : Emulate flashing characters when drawing a Spectrum

screen (see `The Configuration File' for details).

- Slow flash        : Slow down the flashing timer to a third.

(see `The Configuration File' for details).

9. TAPE CATALOGS

----------------

A tape catalog reports the following information per entry:

If the entry represents a normal tape block:

1. (Expanded) Spectrum file name;

2. Type of file: PROG, CODE, A-AR, N-AR, HEAD, DATA, ????, PURE DATA;

3. Depending on the type:

PROG (Program)                   : Auto-start line or ----- if none;

CODE (Code)                      : Startaddress of the code;

A-AR (Alphanumerical Array)      : -

N-AR (Numerical Array)           : -

???? (Unknown header block)      : The Type byte;

HEAD (Headerless block)          : The Flag byte;

DATA (Data block after a header) : The Flag byte;

PURE DATA                        : -

4. Length of the block.

Otherwise:

Short description of the entry type, in blue.

Entries with variable data sizes have their length at the end of the line.

If a line is written in red, there's an error in the entry (such as a bad parity

byte). When you view detailed info, the exact error is written in red too.

If a (data) line is written in bright red, the entry will loose data when saved

(for example, a 'Normal Speed' block with an incomplete byte at the end looses

that incomplete byte when saved, unless saved as VOC). When you view detailed

info, the exact part is written in bright red too.

10. USING A CUSTOM PALETTE

--------------------------

If you dislike the choises I made for the colors of the Spectrum, or you are

unable to see everything due to your (monochrome) monitor, or you just want to

see how weird Spectrum screens (and TAPER itself for that matter) can look, you

can use a custom palette. The colors to be used must be set up in your

configuration file and you must turn `UseCustomPalette' ON.

The values `CustomColorXX' are in Spectrum order and bright colors have 8

added to their number (eg. bright red has number 10).

The values used must be supplied in hexadecimal, preceeded by the tag `0x' for

clearity. Both upper and lower case characters may be used in the number.

For users unfamiliar with the hexadecimal system, you only need to be aware of

the following simple rules:

1. While the decimal has base 10, hexadecimal uses base 16;

2. Digits above 9 are represented by letters starting with 'A', so

`A' = 10, `B' = 11, `C' = 12, `D' = 13, `E' = 14 and `F' is 15.

The numbers themselves have the following format: `BBGGRR', where

`BB' holds the Blue color part,

`GG' holds the Green color part,

`RR' holds the Red color part.

As the chosen video mode has 6 bits/color, the largest allowed number in each

part is 3F. In other words, the color 0x3F3F3F will represent the brightest

possible white.

If you don't like the changes you made and want to try again but forgot the

original values, simply remove the `CustomColorXX' values from your

configuration file, start TAPER, choose `Config' and `Save' to get fresh values.

11. USING A SOUNDCARD

---------------------

When TAPER starts, it automatically checks for a SoundBlaster compatible sound

card. As some soundcards crash on detection functions, TAPER will use the

environment variable `BLASTER' for its values.

Only the 8-bit functionality of the soundcard is used, so the following parts

must be provided:

Axxx - base Address      (xxx usually 220)

Ix   - IRQ line          (x usually 5)

Dx   - 8-bit DMA channel (x usally 1)

If this environment variable is not found, or either one of the above 3 parts

is missing, detection fails.

Otherwise, a short test is done to see if the soundcard responds to requests

at the base address.

You can see whether the card was detected succesfully by pressing the `SysInfo'

button.

12. TROUBLESHOOTING

-------------------

Q: When I return from TAPER to DOS, or when I start a shell from within TAPER,

the screen does not have a cursor, but I see a cursor-like line

somewhere in the void. The font seems to have changed too. What happened?

A: I encountered this problem so far only when using MouseWare from Logitech.

If you use Logitech's MouseWare, ensure that the VCI module `LVESA.OVL' is

loaded and cursor enhancements are off.

To do this, edit your `MOUSEDRV.INI' file. Look for the section `[Display]'

and ensure that a line `VideoModule=C:\MOUSE\LVESA.OVL' is present (or

whereever the `LVESA.OVL' file resides).

Now, if you load your mouse driver (possibly from your `AUTOEXEC.BAT' file),

load it with parameter `NOCENHANCE'.

Q: The mouse makes a mess of the screen! I don't like this at all!

A1:You have a Genius mouse with a driver version 7.02 or below.

Genius mouse drivers v7.02 and below have a different approach for mouse

hiding. If you use one, weird things can happen. Yet, no harm is done by

this to the application nor your computer. Two solutions are available:

1. Get yourself a newer mouse driver, or

2. Turn the option `UseGeniusMouseHiding' on in the configuration file.

A2:You have a combination of an older (ISA) video card and an older mouse

driver, which does not provide the RIL (Register Interface Library).

Be sure that you turned the option `DirectVideoControl' off in `TAPER.INI',

as this disables the RIL.

You can see if the RIL is present when pressing the `SysInfo' button.

If a RIL is present, you will see a line `RIL version : xxxx' in the video

section. If it is absent (or not used), you get `Control : Direct' instead.

You can solve this problem in the following ways:

1. If you are running TAPER in a DOS-box from OS/2, update the box's

parameters, so that compatibility mode is on.

2. Use a newer, fully Microsoft compatible mouse driver, or

3. Add the following line in your `CONFIG.SYS':

DEVICE = EGA.SYS

This device driver also provides the RIL.

Q: When I start TAPER, the screen is cleared, I see a cursor sitting in the

upper left corner, but all activity has stopped. What did I do wrong?

A: You started TAPER while you were in a non-supported video mode (for example

132x44). This causes the graphics library to crash completely. You need a

full reset of the computer to restart. Next time, use the command `MODE 80'

before starting TAPER.

Q: When I have TAPER running, I can't see what's on the screen very well.

Is there anything I can do about it?

A: Yes, you can use a custom palette to create better color contrast.

See `Using a Custom Palette' for details.

Q: I run TAPER from Windows 95/NT. The mouse cursor looks very weird. The mouse

cursor also messes up the screen. What's this?

A: As I said earlier, you shouldn't run TAPER from (any version of) Windows.

Windows 95/NT is the funniest of all. Although it's from Microsoft, the

emulated mouse cursor in a DOS box is absolutely NOT Microsoft compatible!

Talk about weirdos, eh!

You should turn on the `NoLocalMouseHiding' in the configuration file. This

will solve the screen problems, although the mouse will still look weird

(and flickers).

Q: I run TAPER from a Windows (95) DOS-box. When I start it, it complains about

a missing mouse driver. I'm using my mouse without problems within Windows

though!

A: This is a common mistake. Windows and DOS each need their own mouse driver.

The mouse driver from Windows cannot be used within DOS and vice versa.

You should add a line in your `AUTOEXEC.BAT' to activate a mouse in DOS

(or type it on the command line before starting TAPER).

If you upgraded from Windows 3.x to Windows95, the install program from

Windows95 removed this line from your `AUTOEXEC.BAT' (or put `REM' in front

of the line) for no appearant reason, so you should add it back and write a

hate-mail to Bill Gates.

Q: When I run TAPER from DOS, I get XMS version 3.0. But when I run TAPER from a

DOS-box in Windows 3.x, I get XMS version 2.0, although I didn't change my

XMS driver. How's that possible?

A: Windows 3.x uses an internal, enhanced (yeah, right) XMS driver, which

overrides the driver you installed already. This driver is actually A LOT

slower (that's why they invented version 3.0!!!), but should work fine

with TAPER.

Q: My soundcard works with TAPER from DOS, but not from Windows. How's that?

A1:Windows has already allocated the soundcard to another application.

A2:Windows has wrong information regarding your soundcard.

To find out which one it is, close all applications, start a fresh DOS box

and start TAPER in it. If it now works, the answer is A1.

Otherwise it's A2. Click the `SysInfo' button in TAPER to see if the

soundcard has succesfully been detected.

Q: When I play the tape through my soundcard, the pointer bar is always ahead;

A: This is correct. The pointer bar is updated when the next block is going to

be written to the DMA buffer. As the actual playing is slower than the

writing, it is very likely that the next block can be written while the

previous one is still playing.

Q: While playing the tape through my soundcard, I hear double-tones 2 or more

times per second.

A: You are using EMM386 with the 'I=B000:B7FF' option. This provides extra

Upper Memory space if the monochrome screen pages are not used.

Stop using this option, as this space is needed.

13. THE CREDITS

---------------

We would like to thank the following people:

- Sir Clive Sinclair for creating such a terrific machine;

- Jeroen J. Kwast of ThunderWare Research Center for his enthusiasm, details

and for being such a critical beta-tester;

- Andrew David Barker for all his help and VOC files to play with;

- Stefano Donati for his great help with SpeedLock timings;

- Claus Jahn for his endless enthusiasm and very detailed beta reports;

- Everyone who downloaded and tested the beta versions of TAPER;

- Getronics Network Services for allowing my Spectrum archive on their net;

- The folks at the Netverksgruppa for maintaining this huge Spectrum archive

at www/ftp.nvg.unit.no and for having TAPER in their archive.

- All those people who maintain WWW pages and archive sites for the Spectrum;

- All those people who made snapshots of games and gave them away!

14. HOW TO CONTACT US

---------------------

If you have suggestions for improvement (bug reports are also welcome... ;)

write an e-mail to the following address:

Martijn van der Heide: mheide@gns.getronics.nl

Martin Kopanske      : kopanske@media-saturn.com

Tomaz Kac            : tomaz.kac@uni-mb.si

Or a `normal', oldfashioned snail-mail letter to:

ThunderWare Research Center

t.a.v. Martijn van der Heide

Daguerrestraat 120

2561 TX  Den Haag

The Netherlands

Enjoy!

Martijn van der Heide

Software Development Engineer

ThunderWare Research Center