|  | # EU Telematics Applications Programme |  |
| --- | --- | --- |

The basic conferencing tools

These tools allow users to hold multimedia conferences over the Internet using the Multicast Backbone (Mbone) to provide efficient communication in an environment not ideally suited to conferencing.  Multimedia conferencing allows work-sharing in addition to audio and video communication.

##### SDR - the Session Directory provides a directory service allowing both the announcement of public multicast sessions and the establishment of private links.  Using the simple interface, users may join any advertised session.  Encrypted announcements provide both authentication and privacy.

Secure Conference Store (SPAR) – provides an alternative, java-based, approach to the announcement of public multicast sessions with security and authentication using a web browser to both setup the sessions and to launch the tools.

RAT - the Robust Audio Tool, was developed at UCL to provide increased resistance to network loss by using a redundant, low-bandwidth audio stream, transmitted along with the primary audio stream.  RAT has since been developed to handle high quality encoding, and stereo.  Work is now proceeding on sound localisation.  RAT provides encryption for confidential conferencing.

RAT -

##### FreePhone - an audio tool, developed by INRIA, has many features similar to RAT, including the provision of redundant encoding techniques and a variety of codecs for use in differing network conditions. It features automated adaptation to varying network conditions.

NTE  - the Network Text Editor, developed at UCL, allows many users to simultaneously edit a text document over the multicast-capable Internet.  It displays individual users’ contributions in different colours.  Any participant may update or delete any other participant’s contribution unless that contribution is locked.  Files may be loaded and saved in text format.  NTE allows encryption of the data. Version 2.0 is IPv6 compatible.

##### Rendez-Vous - developed by INRIA, it uses new approaches to video encoding and process management to optimise the coding and decoding of video.  It integrates both video and audio (the FreePhone tool) in a single interface.

##### The integrated interface was developed for the Remote Language Teaching project (ReLaTe), it has proved popular and effective in a number of similar projects since.  It can display up to eight video images.  The user sees a single window in which are embedded video (vic), audio (RAT) and shared workspace tools (NTE and wbd) between which the user may switch easily.  The image shown is of the Windows95/NT version developed for the SHRIMP project.

Further Information from the Principal Investigator Prof. Peter T Kirstein             kirstein@cs.ucl.ac.uk

http://www-mice.cs.ucl.ac.uk/multimedia/projects/meccano/

Gatewaying tools

We are continuing to develop two gateway tools which allow users to access the Mbone without being directly connected. UTG allows access over N-ISDN or other low bandwidth link and AudioGate allows interconnection between the telephone network and multicast conferences. AudioGate is an intermediate tool which will be superseded by StarGate, capable of allowing H.323 products to join an multicast conference

##### UTG - the UCL Transcoding Gateway, consists of a Server running on a Sun workstation connected to the Mbone and a client running on a Windows 95/NT machine at the end of a unicast link.  The unicast link is typically IP over ISDN-2, but could be any IP link.  The client can connect to the server and receive Session information via SDR; when a session is selected, the server will send the relevant traffic to the client at the rates defined by the client.  In this way a user is able to allocate the bandwidth between the available media for the session.

##### StarGate - from TZI Bremen will allow H.323 terminals to interwork with the MECCANO tools. The initial version of this gateway, AudioGate, allows participation in multicast conferences from a telephone handset. Work is proceeding to extend the audio-only link to the full audio and video connection needed for H.323-IP multicast interworking.

# Conference recording and replay

##### MMCR – the Multimedia Conference Recorder from UCL allows the recording of multicast conferences by the storage of the RTP packets received from the network. These stored media streams may be replayed subsequently into another conference or simply reviewed. MMCR allows editing and indexing of stored media and the storage and replay of encrypted conferences.

##### SVDA – Scalable Video Distribution Architecture from ACC provides flexible hierarchical storage, booking, caching, and real-time delivery of stored video, together with indexing and retrieval of multimedia document collections. It also has a video-on-demand interface.

# Monitoring

##### MultiMON - a multicast monitoring tool developed by CRC. This tool monitors a network segment, reports on multicast traffic seen and optionally allows one to view and record RTCP statistics. It is built on a client/server basis, allowing the data collectors (Servers) to be distant from the GUI front-end displays (Clients).

##### RQM - RTP Quality Matrix  - from UCL provides a visual display of the packet loss for recipients of a multicast session by monitoring reception reports received via RTCP. It can also initiate an trace (Mtrace) to provide a detailed report of the multicast con-nectivity between any two participants.

The MECCANO Partners

#### Academic Computer Centre CYFRONET	Shell International Exploration and Production B.V.

#### Communications Research Centre, Canada	TELES AG

#### EUTELSAT	Universitaet Bremen

#### Hewlett-Packard European Laboratories, UK 	University College London

#### Institut National de Recherche en Informatique et en Automatique	Universitaet Erlangen-Nuernberg

#### New Learning AS	Universitaet Freiburg

#### Rechenzentrum Universitaet Stuttgart	Universitaet Mannheim

#### School of Slavonic & East European Studies, University of London	University of Oslo

January 2000