|  | # EU Telematics Applications Programme |  |
| --- | --- | --- |

# Accessing Mbone Sessions over N-ISDN

The gateway described here was developed by Jin Yang as part of the Telematics for Research Project MERCI. The work is continuing in this project.

## Introduction

Two issues have to be addressed to provide conferencing connectivity to users over N-ISDN or other low speed circuits. The first is concerned with the mismatch of link speeds. In existing applications, each sender transmits at the same rate to all receivers. With heterogeneous receivers, the source would have to run at a rate that matches the most constrained receiver. When N-ISDN is used, the available bandwidth is limited to 64 or 128 Kbps. Currently most MBone transmissions use a target rate of 128 Kbps for video and 64 Kbps for audio, precluding simple bridging of MBone sessions across 128 Kbps ISDN lines. Thus some kind of rate control, by audio transcoding or video frame rate reduction, becomes necessary. The second issue concerns network configuration. Multicast routing is not yet ubiquitous, so there are sites that wish to participate in MBone sessions but cannot do so. This applies in particular to workstations situated at the end of ISDN lines running IP (usually PPP) over ISDN. Such workstations often do not wish to run full Multicast routing, or cannot do so if they are running any Microsoft operating system without a local Multicast router. For them, an alternative solution is required which this new Gateway provides.

## The network connection

A PC running Windows 95 connects to an Ascend ISDN router which runs TCP/IP over PPP. The Ascend supports multi-link PPP. Thus, in conjunction with the ISDN Accelerator Pack at the PC end, two B-channels can be bound to provide a 128 Kbps PPP link.

## SDR via relay

The session directory tool sdr serves two purposes in the gateway:

- it listens to and displays session announcements

- it allows the remote user to invoke the relevant media tools to join a session.

The sdr relay listens to session announcements, which it caches and passes to the remote PC. On the PC side, the relayed SAP packets are sent to the well-known multicast address for session announcement and are then picked by a locally executed sdr.

## Remote monitoring and control of the gateway

The gateway agents (audio mixer, video filter and shared workspace relays) need to be controlled by users behind the ISDN link. A GUI allows users to control the relay agents.

### Client User interface

Conference Relay, the top-level user interface, is started by sdr when a user joins a session from sdr on the PC or can be started independently.

General functionality

Join will initialise the audio mixer or video filter on the server and start the relevant local media tools. The tools (vic, rat, etc) run point-to-point from the PC to the remote server.

Pause will cause the audio mixer or video filter to stop forwarding packets.

Audio Mixer Control

Currently four encoding/decoding methods are supported by the audio mixer/transcoder. Each encoding transmits the audio stream at a different bit rate. Approximately, the bit rates required are PCM -64 Kbps, DVI - 32 Kbps, GSM - 13.2 Kbps and LPC - 5.8 Kbps. The user chooses the encoding and the corresponding bandwidth for the audio session is displayed. The default encoding is DVI.

Video Relay Control

For video bandwidth, users have control over the individual video streams.

There are three modes for the reception of video streams on the remote host (see the video part of the Conference Relay window above):

- Share b/w. Video streams from all participants share the bandwidth allocated. This is the default mode; if the bandwidth for video is 60 Kbps and there are three participants transmitting video streams, each stream will be filtered so that its bit rate will not exceed 20 Kbps.

- Pass through. Choosing pass-through means no rate control is required. All video streams from the Mbone are forwarded to the client without modification.

- Control b/w individually. This setting means that the bit rate for each stream is to be set separately. To set bit rate for a specific stream, a further user interface is needed.

The main section of this interface is a window showing information about all active senders in the current session. It allows the user to set bandwidth using the scrollbar, up to the total bandwidth allocated for video in the session.

Whiteboard & Text Relay Controls

The bandwidth for both whiteboard and text is fixed at 12 Kbps at present.

### The configuration menu

The configuration menu sets the configuration options for the whole session.

The total bandwidth is that available for both audio and video over the ISDN link. The transmitting rate for the gateway server to feed from the PC to the Mbone is set in the top-level interface (see Conference Relay window above).

Operation modes. Local mode means the multimedia tools are running locally at the same host as the interface. Remote allows the relay service to be made available to another unicast host.

## Local conference bus

The local conference bus was proposed by Steve McCanne and used by the LBL media tools to provide a mechanism for co-ordination among separate tools. This type of mechanism is used in the gateway: the video filtering engine and audio relay engine interact with their control interfaces through a conference bus.

Currently it is used only for this message exchange between audio/video engines and their control interfaces. It will be further deployed to co-ordinate different engines (e.g. audio and video engines) or to control multiple copies of the same engine (e.g. 3 video engines for 3 clients).

# Contact Details:

#### Peter T Kirstein            kirstein@cs.ucl.ac.uk

Department of Computer Science

University College London

Gower Street

London WC1E 6BT

Phone: +44 (0)171 380 7286

Fax: +44 (0)171 387 1397

September 1998