FTP Software Packet Driver FOR USE WITH

8003/8013 Family LAN ADAPTERS

RELEASE INFORMATION

Version 2.02        24 July 1991

The purpose of this document (RELEASE.DOC) is to explain

the version history, bug fixes and any known problems with

the Packet Driver for 8003/8013 Family LAN Adapters.

**Version 2.02    24 July 1991 (Current Release) **

This version includes support for the following WD Ethernet cards.

1.	WD 8013 EP/A

2.	WD 8013 WP/A

This version fixes the following bugs found in version 2.01.

1.	Interrupts 10, 11 and 15 now work with the

Wd 8013 EB/EBT.

2.	Loading, unloading, and then reloading the

Packet Driver now works.

**Version 2.01    25 October 1990 **

This version contains a modification which will allow Novell's

NetBIOS to be loaded after IPX.COM and before the packet driver

(8003PKDR). Previously 8003PKDR would function only if loaded

prior to Novell's NetBIOS.

This version contains modifications for the WD83C690 Network

Interface Controller. This version or later versions must be

used with adapters which contain the WD83C690.

NOTE

Use of adapters which contain the WD83C690 with previous

versions of this driver may lead to data integrity problems.

**Version 2.00    August 1990 (Current Release)**

This release of the Packet Driver conforms to the FTP

Software's Packet Driver Specification v1.09. It can be

loaded as a "Stand-Alone" driver as well as be loaded on

top of the NetWare Shell (IPX.COM) driver sharing the LAN

adapter with the NetWare Shell. This driver has been tested

with FTP's PC/TCP, Wollongong's WIN/TCP, and NCSA's TELNET

software packages. The INSTALL.DOC contains information

on installing with each these software packages.

**Version 1.00    July 1990 **

This is the first official release of this PC/TCP Packet

Driver v1.00. It conforms to the FTP Software Packet

Driver Specification v1.08, and implements the "basic"

functions.  The driver runs on top of the Netware Shell

driver (IPX.COM), and the two drivers share one adapter to

transfer both TCP/IP packets and Netware packets.  In

order to use this driver, you must have the Netware IPX.COM

generated with driver (.OBJ & .LAN) release v3.07 files or

later. This driver can not be used with the StarLAN driver

using the Novell/AT&T StarLAN packet format.

Licensed by Western Digital for use by its customers only.

Western Digital is a registered trademark of Western Digital

Corporation. Other trademarks mentioned herein belong to

their respective companies.

Copyright (c) 1990 Western Digital Corporation

All Rights Reserved.#####################################################################